| Surname |  | Group No |  | Personal Tutor |
| --- | --- | --- | --- | --- |
| First Name |  | Discipline |  |  |

UNIVERSITY OF BIRMINGHAM

School of Engineering

Experiment AM1.1-Motion under variable acceleration

Make sure you know where the nearest fire exits are and what the evacuation procedure is.

Make sure you know where the nearest fire-aiders are located and what their phone numbers are.

maintain a steady footing.

Safety:

Introduction:

Newton’s second law states that the acceleration of a constant mass objects equals the force acting on the object divided by the mass of the object. The force on the object could be constant or vary as a function of the behaviour of the Object; for example it may be a function of the object velocity or position.

The aims of this experiment are to:

- establish an analysis and investigate the difference between the theoretical and experimental conditions

- find the Dynamic Coefficient of Friction.

In this lab, a small mass, m, will be connected to the Dynamics Cart by a string as shown in the figure 1. The string passes over a pulley at the edge of the table so that as the mass falls, the cart will accelerate over the table surface. Assuming that the string is light, inextensible and initially taut, both the falling mass and the dynamics cart should have the same magnitude of acceleration. The resulting magnitude of acceleration for this system will be determined experimentally and this value will be compared to the acceleration predicted by Newton’s Second Law.

The following equipment is required for the experiment:

m

Figure 1 Schematic drawing of the system

– Dynamics Cart

MC

– Pulley and pulley clamp

– Mass set

– Stopwatch

– String

– Block

– Balance

Theory:

The cart will be released from rest and allowed to accelerate over a distance d. There are three forces act on the cart which are –Tension force T, Normal force N (counter balance of weight), Friction force fK. The acceleration can be predicted by Newton’s Second Law.

Assuming that the table top is accurately horizontal (i.e. level),

⅀F=Ma→⅀FH=mg-T=maH      (Eq.1)

⅀FC=T-fk=MCa    (Eq.2)

→atheoretical =mMTg    (Eq.3)

where MT  is the total mass of the system , μk is kinetic friction coefficient and T is tension force applied to the Cart .

Here we consider the motion with no friction. The acceleration value is achieved from the theoretical calculation.

An experimental value for the acceleration a of the cart can be determined from:

d=12at2  →aExperimental=2dt2         (Eq.4)

where d is the maximum distance which the Cart travel, which is 0.9 meter.

The Friction force based on the obtained experimental acceleration would be calculated as follows:

Fnet=MCa→T-fk=MCa→fk=T-MCa    (Eq. 5)

where a is the experimental acceleration.

The gradient of the best fitted line to the Friction force vs Normal force will indicate the frictional coefficient.

Results:

Fill the tables in next page and calculate the required values as well as drawing the Free Body Diagram. Plot the New tension force based on the experimental acceleration a versus the Cart normal force Fnet. Estimate the best fit line to these plots and find the Kinetic Friction coefficient.

Report:

In your report you should include the experimental data, brief explanation of the theory, free body Diagram of the system and a graph of the new Friction force based on the experimental acceleration against the Cart normal force. The frictional force graph should include a line of best fit. Furthermore, the obtained results should be analysed and discussed. Examine the accuracy of various measurements and estimate the level of confidence you can place on the results.

Can you think of any systematic errors that would affect your results? Explain how each would affect your results.

Comment on the results obtained: 1. Comparison of Experimental and Theoretical Acceleration The experimental acceleration was consistently lower than the theoretical acceleration  across all three tests. This is expected because the theoretical calculation assumes an ideal, frictionless system. In reality, kinetic friction between the cart wheels and the track, as well as friction in the pulley axle, opposes the motion, resulting in a lower net force and consequently lower acceleration.

2. Analysis of Errors The percentage difference between theoretical and experimental values ranged from approximately 6% to 40%. A distinct trend is observable: the percentage difference is significantly higher when the hanging mass is small (10g).

- Example: In Test 2, the 10g mass yielded a 39.48% difference, whereas the 40g mass yielded only a 7.68% difference.

- Reasoning: The frictional force is relatively constant for a given cart mass. When the driving force (provided by the hanging mass) is small, the friction force constitutes a larger proportion of the total forces, leading to a larger deviation from the theoretical "frictionless" prediction. As the driving force increases (40g), the friction becomes less significant relative to the total force, and the experimental results align closer to the theory.

3. Systematic Errors Several systematic errors likely affected the results:

- Friction: As noted, this is the primary systematic error causing all experimental values to be lower than theoretical predictions.

- Level of the Track: If the track was slightly inclined upwards, gravity would oppose motion, further reducing acceleration. If inclined downwards, it would artificially increase it.

- String Mass: The theory assumes a massless string. In reality, the string adds a small amount of mass to the system, slightly affecting inertia.

- Reaction Time: While digital timing gates were likely not used (based on the stopwatch reference), manual timing introduces human reaction error, though averaging multiple runs helps mitigate this.

4. Kinetic Friction Coefficient  By plotting the Friction Force  against the Normal Force , we obtained a linear relationship. The gradient (slope) of the line of best fit represents the coefficient of kinetic friction.

- Calculated Slope : Approximately 0.001 (or 0.0008).

- This extremely low value indicates that the dynamics cart has high-quality, low-friction bearings. However, the presence of a non-zero friction force confirms that friction cannot be entirely neglected in accurate physical models.

Experiment AM1.1 - Completed Tables

Below are the calculated values for your lab report based on the provided data image.

Constants Used

- Distance (d): 0.9 m

- Gravity (g): 9.81 m/s²

- Mass of Cart (Estimated): 0.5 kg (This is standard for these experiments and fits the physics of your data).

Test 1 - Without added mass

(Cart Mass = 0.5 kg)

| Mass (g) | Average Time (s) | Average Velocity (m/s) | Experimental Acc (m/s2) | Theoretical Acc (m/s2) | Difference % | Friction Force (N) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 3.421 | 0.263 | 0.154 | 0.192 | 19.9% | 0.0195 |
| 20 | 2.283 | 0.394 | 0.345 | 0.377 | 8.49% | 0.0166 |
| 30 | 1.8825 | 0.478 | 0.508 | 0.555 | 8.51% | 0.0251 |
| 40 | 1.63 | 0.552 | 0.677 | 0.727 | 6.78% | 0.0266 |

Test 2 - With 500g added mass

(Total Cart Mass = 1.0 kg)

| Mass (g) | Average Time (s) | Average Velocity (m/s) | Experimental Acc (m/s2) | Theoretical Acc (m/s2) | Difference % | Friction Force (N) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 5.537 | 0.163 | 0.059 | 0.097 | 39.48% | 0.0388 |
| 20 | 3.2795 | 0.274 | 0.167 | 0.192 | 13.02% | 0.0255 |
| 30 | 2.6405 | 0.341 | 0.258 | 0.286 | 9.67% | 0.0284 |
| 40 | 2.2735 | 0.396 | 0.348 | 0.377 | 7.68% | 0.0302 |

Test 3 - With 1 kg added mass

(Total Cart Mass = 1.5 kg)

| Mass (g) | Average Time (s) | Average Velocity (m/s) | Experimental Acc (m/s2) | Theoretical Acc (m/s2) | Difference % | Friction Force (N) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 6.4395 | 0.140 | 0.043 | 0.065 | 33.21% | 0.0326 |
| 20 | 4.096 | 0.220 | 0.107 | 0.129 | 16.92% | 0.0331 |
| 30 | 3.22 | 0.280 | 0.174 | 0.192 | 9.68% | 0.0288 |
| 40 | 2.741 | 0.328 | 0.240 | 0.255 | 5.97% | 0.0232 |

Summary Table: Calculated Friction Forces (N)

(For the table at the bottom of Page 4)

| Normal Forces | 10 g hanged mass | 20 g hanged mass | 30 g hanged mass | 40 g hanged mass |
| --- | --- | --- | --- | --- |
| 0.5 kg X g (Test 1) | 0.0195 | 0.0166 | 0.0251 | 0.0266 |
| 1.0 kg X g (Test 2) | 0.0388 | 0.0255 | 0.0284 | 0.0302 |
| 1.5 Kg X g (Test 3) | 0.0326 | 0.0331 | 0.0288 | 0.0232 |

Graphing Data