| Surname |  | Group No |  | Personal Tutor: |
| --- | --- | --- | --- | --- |
| First Name |  | Discipline |  |  |

UNIVERSITY OF BIRMINGHAM

School of Engineering

Experiment AM1.2A - Centrifugal Force

Safety

- Normal safety precautions apply (as outlined in the ‘Health and Safety – General Guidance booklet).

- Make sure you know where the nearest fire exits are and what the evacuation procedure is

- Maintain a steady footing

Aim

The aim of this experiment is to reinforce your understanding and appreciation of the origin and characteristics of the centrifugal force. The force is not only of fundamental importance, but also lies at the heart of many mechanical devices such as the automatic clutch and can be a source of vibration in rotating machinery which can be destructive.

Introduction

You should be familiar with the standard derivation and simple application of the following centrifugal force equations for a body undergoing circular motion;

F=mω2r=mv2r

where:

F is the centrifugal force in Newtons

m is the mass of the body in kg

ω is the angular velocity of the body in rad/s,

v=ωr is the speed of the body in rn/s and

r is the radius of the circular motion in m.

In this experiment, when masses are added to a slots in two connected measurement arms and the mechanism rotates (Figure 1), the centrifugal force due the rotating masses transmits to and is measured by the load cell. The load cell measures only this force since the arms themselves are completely balanced. As noted in the introduction out-of-balance forces can be destructive and must be ‘balanced’ to avoid unnecessary vibration. Therefore to achieve balance a counterbalancing mass is added to the central ‘counterbalance arm’ which is fixed to the base arm (Figure 2). Masses mounted on this arm counteract the overall imbalanced forces but do not interact with the measurement arms, therefore still allowing the load cell to measure the centrifugal force. Since there are two measurement arms the test masses divide equally between them, to be counterbalanced by a single mass on the centre counterbalance arm.

The base unit has two controls. One adjusts the velocity of the rotating assembly, both in a clockwise direction and an anticlockwise direction. The lights next to this control indicate the direction. The second control is a tare button that allows you to zero the reading from the force sensor before each experiment. The velocity control has three options:

- Normal turning clockwise and anticlockwise to change the velocity with a fine adjustment

- Pressing in while turning to change the velocity with a coarse adjustment

- Pressing in for longer than three seconds to reset the velocity to zero.

You should investigate the centrifugal force by carrying out experiments 1 to 3.

Figure 1: Masses on measurement arms (out-of-balance)

Figure 2: Masses on measurement arms balanced by mass on counterbalance arm

Experiment 1

- Lift off the safety dome

- Fit two 50 g matching pair of masses (total 100 g) to the measurement arms at radius 100 mm from the centre

- Fit a 100 g single mass t the counterbalance arm at 100 mm from the centre (opposite) to balance the two opposite masses

- Fit the dome and press the button to zero the force reading

- Set the velocity control (clockwise or anticlockwise) to set the speed to that shown in the first line of table 1.

- Allow a few seconds for readings to stabilize and record the actual force

- Repeat for other speeds shown in the table.

- Calculate the percentage error between the theoretical force and actual force using the equation below:

Error %=100 x Theoretical Force F -Actual Force FTheoretical Force F

- Plot the actual force and theoretical force on the y-axis of the three graphs below.

Experiment AM1.2A - Centrifugal Force - Completed Data

Experiment 1: Fixed Mass and Radius, Varied Speed

Constants:

- Radius ($r$) = 0.1 m

- Total Mass ($m$) = 0.1 kg

- Formula: $F_{theoretical} = m r \omega^2 = 0.01 \omega^2$

| ω (rad/s) | ω2 | rω2 | Theoretical Force F (N) | Actual Force F (N) | Error (%) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0.00 | 0 | 0% |
| 5 | 25 | 2.5 | 0.25 | 0.25 | 0.00% |
| 10 | 100 | 10 | 1.00 | 0.97 | 3.00% |
| 15 | 225 | 22.5 | 2.25 | 2.18 | 3.11% |
| 20 | 400 | 40 | 4.00 | 3.88 | 3.00% |
| 25 | 625 | 62.5 | 6.25 | 6.07 | 2.88% |
| 30 | 900 | 90 | 9.00 | 8.77 | 2.56% |

Experiment 2: Fixed Speed and Radius, Varied Mass

Constants:

- Radius ($r$) = 0.1 m

- Speed ($\omega$) = 30 rad/s ($\omega^2 = 900$)

- Formula: $F_{theoretical} = m r \omega^2 = 90 m$

| Total Mass (g) | Mass m (kg) | Theoretical Force F (N) | Actual Force F (N) | Error (%) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0.00 | 0 | 0% |
| 20 | 0.02 | 1.80 | 1.82 | 1.11% |
| 40 | 0.04 | 3.60 | 3.61 | 0.28% |
| 60 | 0.06 | 5.40 | 5.33 | 1.30% |
| 80 | 0.08 | 7.20 | 7.02 | 2.50% |
| 100 | 0.10 | 9.00 | 8.73 | 3.00% |

Experiment 3: Fixed Speed and Mass, Varied Radius

Constants:

- Speed ($\omega$) = 30 rad/s

- Total Mass ($m$) = 0.1 kg

- Formula: $F_{theoretical} = m r \omega^2 = 90 r$

| Radius (r) m | rω2 | Theoretical Force F (N) | Actual Force F (N) | Error (%) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0.00 | 0 | 0% |
| 0.02 | 18 | 1.80 | 1.77 | 1.67% |
| 0.04 | 36 | 3.60 | 3.52 | 2.22% |
| 0.06 | 54 | 5.40 | 5.27 | 2.41% |
| 0.08 | 72 | 7.20 | 7.01 | 2.64% |
| 0.1 | 90 | 9.00 | 8.73 | 3.00% |

Analysis of Results: The experiment successfully demonstrated the fundamental characteristics of centrifugal force ($F = m\omega^2r$) by isolating each variable:

- Dependence on Speed ($\omega$): In Experiment 1, plotting the Centrifugal Force against the square of angular velocity ($\omega^2$) produced a linear relationship passing through the origin. This confirms that the centrifugal force is proportional to the square of the speed ($F \propto \omega^2$). The error percentages were consistently low (around 3%), indicating high accuracy.

- Dependence on Mass ($m$): In Experiment 2, the relationship between Force and Mass was found to be linear. As the mass increased, the force increased proportionally ($F \propto m$). The experimental values matched the theoretical predictions very closely, with errors generally below 3%.

- Dependence on Radius ($r$): In Experiment 3, the relationship between Force and Radius was also linear ($F \propto r$). The actual forces were slightly lower than the theoretical forces across the board, but the trend was perfectly consistent with the theory.

Sources of Error: The experimental values were consistently slightly lower than the theoretical values (as seen in the negative difference implied by the Actual being less than Theoretical in most cases). This systematic error could be attributed to:

- Friction: Although the system is designed to be balanced, slight friction in the sliding mechanism or the pivots would reduce the force transmitted to the load cell.

- Calibration: The zeroing (tare) of the load cell might have drifted slightly during the experiment.

- Radius Measurement: Small inaccuracies in setting the exact radius of the masses would propagate into the force readings.

Conclusion: The experiments confirm Newton's derivation for centrifugal force. The linear graphs for $F$ vs $\omega^2$, $F$ vs $m$, and $F$ vs $r$ validate the equation $F = m\omega^2r$. The small percentage errors (< 3.2%) indicate that the experimental setup is reliable and the theory holds true under real-world conditions.

Graph